SAFETY DATA SHEETMEL

SECTION 1: IDENTIFICATION

Product Identification: Clear W/B Urethane Part B

Product Code: Oneaqua W/B hardener

Recommended Use: Coatings

Information Phone: 260-582-9104

Preparation Date: 06/04/2017

Last Revision Date: 02/10/2021

SECTION 2: HAZARD(S) IDENTIFICATION

Hazard Classification(s): Category 4 Acute toxicity –inhalation, Category 1 Skin Sensitizer, Category 3 (Respiratory System) Specific target organ toxicity-single exposure

Signal word: Danger

Hazard Statements:

Toxic if swallowed

Causes skin irritation.

Causes eye irritation.

May cause drowsiness or dizziness.

May be fatal if swallowed and enters airways.

Flammable liquid and vapor

Pictograms:

Precautionary Statements:

Do not handle until all safety precautions have been read and understood.

Do not eat, drink or smoke when using this product.

Do not breathe dust/fume/gas/mist/ vapors/spray.

Wash thoroughly after handling.

Wear protective gloves/protective clothing/eye protection/face protection.

Keep away from heat/sparks/open flames/hot surfaces. – No smoking

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

| Ingredient Name | CAS Number | % wt. |
| --- | --- | --- |
| 1,6-Diisocyanato-Hexane, Homopolymer | 28182-81-2 | 84.4 |
| Methoxybutyl Acetate | 4435-53-4 | 13.3224.5 |
| NNDimethylcyclohexylamine | 98-94-2 | 2.00 |
| Hexamethylene Diisocyanate | 822-06-0 | .29 |

4.5% of the mixture consists of ingredient(s) of unknown acute toxicity.

SECTION 4: FIRST-AID MEASURES

EYES:  Flush thoroughly with water for at least 15 minutes, lifting upper and lower lids occasionally.  Get immediate medical attention.

SKIN:  Wash exposed areas with soap and water, remove contaminated clothing.  If irritation occurs, get medical attention.

INGESTION:  Do not induce vomiting, keep victim warm and quiet.  Aspiration of material into lungs due to vomiting can cause chemical pneumonitis which can be fatal.  Get immediate medical attention.
INHALATION:  Move victim away from source of exposure and into fresh air.  If symptoms of exposure develop, get medical attention.

Medical conditions generally aggravated by exposure: Asthma, other respiratory disorders (bronchitis, emphysema, bronchial hyperactivity), skin allergies, eczema.

SECTION 5: FIRE-FIGHTING MEASURES

Flash Point:  60°F	METHOD USED: TCC

Flammable Limits in Air by Volume - Lower: 1.2%	Upper:  19%

Extinguishing Media: Foam, Co2, Dry Chemical, water fog spray.

Special hazards: Use self-contained  breathing  apparatus,  use  water  for  cooling  container  to  prevent  pressure  buildup,  auto-ignition  or explosion. Avoid spreading burning liquid with water.

Unusual fire and explosion hazards: Flammable vapors may form explosive mixtures with air. Polyisocyanates may decompose in a fire, yielding toxic gases. At temperatures over 400 deg F.(204 deg C.) polymeric resins can polymerize and decompose causing pressure increases within closed containers leading to possible rupture of the containers.  Cool nearby closed containers with water in the event of fire.

SECTION 6: ACCIDENTAL RELEASE MEASURES

Eliminate sources of ignition.  Provide adequate ventilation.  Prevent liquid from entering sewers or water ways. Absorb with inert material or pump into container.

Personal Precautions, Protective Equipment and Emergency Procedures: Protect against physical damage, keep away from open flame and extreme heat. Keep container closed.

Environmental Precautions: Prevent from spreading or entering into drains, ditches or rivers by using sand, earth, or other appropriate barriers.

Other precautions: Do not take internally.  Avoid prolonged breathing of vapors.

SECTION 7: HANDLING AND STORAGE

Precautions for Safe Handling:

Keep away from heat/sparks/open flames/hot surfaces. – No smoking.

Keep container tightly closed.

Ground/Bond container and receiving equipment

Use explosion-proof electrical equipment.

Use only non-sparking tools.

Take precautionary measures against static discharge.

Wear protective gloves/eye protection/face protection

Conditions for safe storage, including any incompatibilities:

Store in a well-ventilated place. Keep cool.

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

| Ingredient | OSHA PEL |  | ACGIH TLV |  |
| --- | --- | --- | --- | --- |
|  | TWA | STEL | TWA | STEL |
| 1,6-Diisocyanato-Hexane, Homopolymer | NE |  |  |  |
| Methoxybutyl Acetate | Not applicable |  |  |  |
| NNDimethylcyclohexylamine | NE NE |  |  |  |
| Hexamethylene Diisocyanate | NE | NE | .005PPM | NE |

NE = none established

Exposure Controls General Information: The level of protection and types of controls necessary will vary depending upon potential exposure conditions. Select controls based on a risk assessment of local circumstances. Use adequate ventilation to control airborne concentrations below the exposure guidelines/limits. Local exhaust ventilation is recommended.

Personal Protective Equipment

Eye Protection: Safety glasses with side shields

Hand Protection: Polyvinyl acetate.

Body Protection: Chemical resistant gloves/gauntlets, boots, and apron (where risk of splashing).

Respiratory Protection: NIOSH approved self-contained breathing apparatus where TLV is exceeded.

Other protective clothing or equipment

Eye bath and safety shower, uniforms or aprons may be used to avoid excessive contact.

Ventilation: Local exhaust ventilation is recommended. 300 cfm minimum.  Use explosion proof ventilation equipment.

Work/hygienic practices

Wear protective clothing, have adequate ventilation, avoid ignition sources.

Environmental exposure control measures: Local guidelines on emission limits for volatile substances must be observed for the discharge of exhaust air containing vapor.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

Appearance (physical state, color, etc.); Liquid

Upper/lower flammability or explosive limits; no applicable information was found

Odor; Solvent Odor

Vapor pressure; no applicable information was found

Odor threshold; no applicable information was found

Vapor density; >1

pH; no applicable information was found

Relative density; (H2O=1) 1.13

Melting point/freezing point; no applicable information was found

Solubility(ies); Solubility in Water is Negligible

Initial boiling point and boiling range; 320⁰F-415⁰F

Flash point; 60⁰F

Evaporation rate; >1

Flammability (solid, gas); no applicable information was found

Upper/lower flammability or explosive limits; Lower: 1.2%-Upper: 19%

Vapor pressure; 0.0 mm Hg at 70 °F

Vapor density; no applicable information was found

Relative density; 9.43 lb./gl.

Partition coefficient: n-octanol/water; no applicable information was found

Auto-ignition temperature; no applicable information was found

Decomposition temperature; no applicable information was found

Viscosity; no applicable information was found

% Volatile by Volume: 15.327%

% Volatile by Weight: 12.683%

Coating V.O.C.:  1.2 lb./gl.

SECTION 10: STABILITY AND REACTIVITY

Reactivity: no applicable information was found

Chemical Stability:  Stable

Conditions to Avoid: All equipment should be grounded and bonded to reduce static electricity hazard. Use non-sparking tools. keep away from heat open flames, etc.

Incompatibility (Materials to Avoid): Strong oxidizing agents

Hazardous Decomposition or Byproducts: Decomposition may yield carbon dioxide and/or carbon monoxide; smoke

Hazardous Polymerization:  Will not occur.

SECTION 11: TOXICOLOGICAL INFORMATION

Basis for Assessment: Information given is based on product data and knowledge of the components and the toxicology of similar products.

Likely Routes of Exposure: Exposure may occur via inhalation, skin absorption, skin or eye contact, and accidental ingestion.

Acute Effects from Exposure: Excessive  exposures  may  cause  irritation  to  eyes,  nose,  throat,  lungs,  respiratory  tract.

Central Nervous System Effects: Headaches, nausea, dizziness and unconsciousness.

Chronic Effects from Exposure: Repeated and prolonged overexposure of solvents may damage liver, kidneys and central nervous system.

Acute Oral Toxicity: 1,6-Diisocyanato-Hexane, Homopolymer: expected to be acutely toxic (IUCLID)

Acute Dermal Toxicity: 1,6-Diisocyanato-Hexane, Homopolymer: expected to be acutely toxic (IUCLID)

Acute Inhalation Toxicity: Low toxicity: LC50 >20 mg/l h, Rat

Skin corrosion/irritation: Irritating to skin.

Serious eye damage/irritation: irritating to the eyes.

Respiratory Irritation: Inhalation of vapors or mists may cause irritation to the respiratory system.

Respiratory or skin sensitization:  Considered to be a respiratory and skin sensitizer.

Germ cell mutagenicity: Not expected to cause genetic damage

Carcinogenicity: Not classified as a carcinogen.

Reproductive and Developmental Toxicity: Not expected to impair fertility. Not classified as a developmental toxicant.

Specific target organ toxicity - single exposure: High concentrations may cause central nervous system depression resulting in headaches, dizziness and nausea; continued inhalation may result in unconsciousness and/or death.

Specific target organ toxicity - repeated exposure: May cause respiratory irritation.

Aspiration Hazard: Aspiration into the lungs when swallowed or vomited may cause chemical pneumonitis which can be fatal.

None of the chemicals in this product are listed in the National Toxicology Program (NTP) Report on Carcinogens (latest edition) or has been found to be a potential carcinogen in the International Agency for Research on Cancer (IARC) Monographs (latest editions) or found to be a potential carcinogen by OSHA.

SECTION 12: ECOLOGICAL INFORMATION

Basis for Assessment: Information given is based on product data and knowledge of the components and the toxicology of similar products.

Eco-toxicity

Acute (short-term) fish toxicity: Not applicable

Chronic (long-term) fish toxicity: Not applicable

Acute (short-term) daphnia toxicity: Not applicable

Chronic (long-term) daphnia toxicity: Not applicable

Acute (short-term) algae toxicity: Not applicable

Chronic (long-term) algae toxicity: Not applicable

Persistence and degradability: Not applicable

Bioaccumulative potential: Not applicable

Partition coefficient: n-octanol/water: Not applicable

Mobility in soil: Not applicable

SECTION 13: DISPOSAL CONSIDERATIONS

Appropriate disposal / Product

Dispose according to local legislation. Consult the appropriate local waste disposal expert about waste disposal.

Appropriate disposal / Package

Dispose according to local legislation. Handle contaminated packages in the same way as the substance itself.

Additional information

No applicable information was found

SECTION 14: TRANSPORT INFORMATION

UN number: UN1263

UN proper shipping name: Paint

Transport hazard class: III

Packing group, if applicable: PGIII

Environmental hazards: No

Transport in bulk: Not applicable

SECTION 15: REGULATORY INFORMATION

EPA Regulations: Not applicable

SARA 311/312 Codes: Not applicable

SARA Toxic Chemical (40 CFR 372.65): Not applicable

SARA EHS (Extremely Hazardous Substance) (40 CFR 355): Not applicable

SECTION 16: OTHER INFORMATION

Prepared By: Eddie Rangel

Revision Notes: NA

Additional Hazard Rating Systems:

Disclaimer: The information in this MSDS was obtained from sources which we believe are reliable.   However, the information is provided without any representation of warranty, expressed or implied, regarding its accuracy or correctness.