SDS

Material Safety Data Sheet

Urethane Clear Part A

Manufacturer’s Name:					                                             Emergency Tel. No.

One Coat Technologies, LLC	                                                                                         Chemtrec:  800-424-9300

5411 Enterprise Drive

Lansing, MI 48911

Chemical Name and Synonyms:  			Trade Name and Synonyms:

Polyester urethane 			                             Onethane Part A urethane clear top coat

SECTION 1-Content / Ingredients

| Chemical Name | Weight % | CAS Number |
| --- | --- | --- |
| Aliphatic polyester resin/ Polyester Polyol | 40.1 | 67815-82-1 |
| METHYL ETHYL KETONE (VOC) | 17.55 | 78-93-3 |
| BUTYL ACETATE | 7.41 | 000123-86-4 |
| Methyl N-Amyl Ketone | 6.09 | 110-43-0 100 |

SECTION 2-Physical Data

APPEARANCE AND ODOR: low viscosity liquid with ketone solvent odor. BOILING POINT OR RANGE: 279 to 329F
VAPOR DENSITY (AIR = 1): not available
SPECIFIC GRAVITY (H2O = 1): 1.0

EVAPORATION RATE: not available SOLUBILITY IN WATER: negligible

Odor Threshhold: N/A
pH: N/A
Melting point/freezing point: N/A
Vapor Pressure: N/A
Auto Ignition Temperature: N/A
Partition Coefficient: n-octanol/water: N/A Decomposition Temperature: N/A

SECTION 3-Fire and Explosion Data

Seta Flash

EXTINGUISHING MEDIA:

Foam, alcohol foam, CO2, dry chemical, water fog.

SPECIAL FIRE FIGHTING PROCEDURES:

Do not enter confined fire area without full bunker gear including a positive pressure NIOSH approved self-contained breathing apparatus. Cool all fire exposed containers with water. Minimize contact with material.
UNUSUAL FIRE AND EXPLOSION HAZARDS:
Closed containers may explode when exposed to extreme heat. Solvent vapors may be heavier than air. Under conditions of stagnant air, vapors may build up and travel along the ground to an ignition source which can result in flash back to the source of the vapors. Toxic vapors could be evolved from the combustion of this material.

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:

Remove all sources of ignition and ventilate the area. Wear appropriate protective equipment such as vapor cartridge or air supplied respirator when necessary. Dike and absorb the material with absorbent such as clay and place in disposal containers.

HANDLING AND STORAGE

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE:

Store in cool dry area. Seal all partially used containers. Wash with soap and water before eating, drinking, smoking or using the toilet facilities. Mixed materials contain the hazards of all the components, therefore, read the MSDS’s of all the components prior to using the material. Properly label all containers.
OTHER PRECAUTIONS:

Avoid all skin contact. Avoid breathing vapors generated from the material. Observe conditions of good general hygiene and safe working practices. Contaminated leather articles cannot be cleaned and must be discarded if contaminated with this product. Wash all contaminated clothing prior to the reuse thereof. Supply appropriate ventilation or engineering controls prior to usingthis product.

SECTION 4-Hazzardous Information

Hazard Overview
GHS Classification: Flammable liquid category 3, Specific target organ toxicity following repeated exposure category 1, Specific target

organ toxicity following repeated exposure category 2, Aspiration hazard category 2, Acute oral toxicity category 4, Acute dermal toxicity category 4, Skin corrosion category 2, skin sensitizer category 1B, Serious eye irritation category 2A, Acute toxicity inhalation category 4, Carcinogenicity category 2, Acute hazard to aquatic environment category 3
GHS Label Elements and Precautionary Statements:
Label Elements: Flame, Health hazard, Exclamation Mark
Hazard Statements:
Warning: Flammable liquid and vapor.
Warning: May cause damage to organs (liver, kidney, nervous system, respiratory system and lungs) through inhalation
Warning: May cause damage to organs (liver, kidney, nervous system, respiratory system and lungs) through prolonged or repeated exposure Warning: May be harmful if swallowed and enters airways.
Warning: Harmful if swallowed
Warning: Harmful in contact with skin
Warning: Causes skin irritation
W arning: May cause an allergic skin reaction
Warning: Causes serious eye irritation
Warning: Harmful if inhaled
Warning: Suspected of causing cancer
Harmful to aquatic life.
Risk Phrases:
R20 Harmful by inhalation
R21/22 Harmful in contact with skin and if swallowed
R25 Toxic if swallowed
R36/37/38 Irritating to eyes, respiratory system and skin
R42/43 May cause sensitisation by inhalation and skin contact
R45 May cause cancer
R52 Harmful to aquatic organisms
R66 Repeated exposure may cause skin dryness or cracking
Precautionary statements:
P102 Keep out of reach of children.
P103 Read label before use
P202 Do not handle until all safety precautions have been read and understood.
P210 Keep away from heat/sparks/open flames/hot surfaces. — No smoking.
P233 Keep container tightly closed.
P234 Keep only in original container.
P261 Avoid breathing dust/fume/gas/mist/vapours/spray.
P262 Do not get in eyes, on skin, or on clothing.
P264 W ash skin thouroughly after handling.
P270 Do not eat, drink or smoke when using this product.
P273 Avoid release to the environment.
P280 Wear protective gloves/protective clothing/eye protection/face protection.
P284 Wear respiratory protection.

HMIS HAZARD CLASSIFICATION
HEALTH: 2 FLAMMIBILITY: 3 REACTIVITY: 0 PERSONAL PROTECTIVE EQUIPMENT: G

POTENTIAL HEALTH EFFECTS
EYES:
May cause corneal damage if left untreated which is slow to heal but usually reversible. SKIN:

SECTION 5-Health Hazard Data

This information is based on our present knowledge. However, this shall not constitute a guarantee for any specific product features and shall not establish a legally valid contractual relationship.

May cause irritation or allergic response. May cause defatting, dryness, cracking, rash or redness or dermatitis.

SKIN ABSORPTION:

Solvents can penetrate the skin causing effects similar to those for acute inhalation symptoms.

INGESTION:

Can cause irritation to the digestive tract including sore throat, abdominal pain, nausea, vomiting and diarrhea. Vomiting may Cause Aspiration of solvents resulting in chemical pneumonitis.
INHALATION health risks and symptoms of exposure:
Solvent vapors are irritating to the eyes, nose and throat and respiratory

tract resulting in dryness of the throat and tightness in the chest. Other symptoms include headache, nausea, narcosis, fatigue and loss of appetite.
HEALTH HAZARDS (ACUTE AND CHRONIC):
Chronic Exposure to organic solvents has been associated with various neurotoxic effects including brain damage, nervous system damage or death. Prolonged vapor contact may cause conjunctivitis. Chronic inhalation may also include loss of memory, loss of intellectual ability and loss of coordination. Corneal damage is possible but usually reversible. Repeated Exposure to solvents can cause anemia, liver abnormalities, kidney damage or cardiac abnormalities.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE:

Respiratory conditions or other allergic response.

CARCINOGENICITY

OSHA: NO NTP: NO IARC: yes

ADDITIONAL CARCINOGENICITY INFORMATION:

SECTION 6- EXPOSURE CONTROLS/PERSONAL PROTECTION

RESPIRATORY PROTECTION:

Use a NIOSH approved respirator as required to prevent over-exposure to vapor in accordance with 29 CFR 1910.134. Use a positive pressure respirator when airborne concentrations are not known or if exceeding TLV’s or if working in a confined space. Always consider the hazards from all components in the mixed material state.
VENTILATION :

Exhaust ventilation sufficient to keep the airborne concentrations of the solvents and other hazardous materials below the toxic level concentrations.
PROTECTIVE GLOVES:
Impervious gloves – neoprene or rubber.

EYE PROTECTION:

Splash goggles or glasses with side shields. If the environment warrants, a full face shield should be employed.

OTHER PROTECTIVE CLOTHING OR EQUIPMENT:

Wear body covering clothing and other coverings as necessary such as an apron and appropriate footwear to avoid contact.

WORK HYGIENIC PRACTICES:

Observe good general hygienic practices.

SEE SECTION THREE FOR OCCPATIONAL EXPOSURE LIMIT VALUES.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

APPEARANCE AND ODOR: low viscosity liquid with ketone solvent odor. BOILING POINT OR RANGE: 279 to 329F
VAPOR DENSITY (AIR = 1): not available
SPECIFIC GRAVITY (H2O = 1): 1.0

EVAPORATION RATE: not available SOLUBILITY IN WATER: negligible

Odor Threshhold: N/A
pH: N/A
Melting point/freezing point: N/A
Vapor Pressure: N/A
Auto Ignition Temperature: N/A
Partition Coefficient: n-octanol/water: N/A Decomposition Temperature: N/A

Section 7-Spill or Leak Procedures

Steps to be taken in case material is released or spilled

Eliminated all sources of ignition and ventilate area.  Dike area to contain spill.  Use absorbent material to soak up spill.  Shovel or sweep into container for disposal.  Wash spill area with detergent solution.

Waste Disposal Method
WASTE TREATMENT METHODS

PRODUCT/PACKAGING DISPOSAL : Dispose according to local legislation. Consult the appropriate local waste disposal expert about waste disposal. Handle contaminated packages in the same way as the substance itself.

WASTE TREATMENT OPTIONS : Waste treatment options should be in accordance with local and state guidelines.

WASTE CODE/WASTE DESIGNATIONS ACCORDING TO EWC/AVV :  F003  The following spent non-halogenated solvents: Xylene, acetone, ethyl acetate, ethyl benzene, ethyl ether, methyl isobutyl ketone, n-butyl alcohol, cyclohexanone, and methanol. See EPA Hazardous Waste Code for more information.

F005 Toluene, methyl ethyl ketone, carbon disulfide, isobutanol, pyridine, benzene, 2-ethoxyethanol, and 2-nitropropane.See EPA Hazardous Waste Codes for more information.

EWC 801 wastes from the manufacture and removal of paint and varnish.

RECOMMENDATIONS :  Do not re-use empty containers. Empty containers should be taken for local recycling, recovery or waste disposal. Emptied containers may retain hazardous residue and explosive vapors. Keep away from heat, sparks and flames. Do no cut, puncture or weld on or near emptied containers.

SAFE HANDLING : Since emptied containers retain product residue, follow label warnings even after container is emptied. Wear personal protective equipment inclusive of safety glasses, respirator, gloves, apron, or other equipment as required.

SECTION 13: Transport Information

DOT (DEPARTMENT OF TRANSPORTATION)

TECHNICAL NAME : Paint

UN NUMBER : UN1263

UN PROPER SHIPPING NAME : Paint

TRANSPORT HAZARD CLASS : 3

PACKING GROUP : PGIII

MARINE POLLUTANT : Not considered a marine pollutant.

SPECIAL PRECAUTIONS : Shipping instructions and descriptions may vary based on mode of transport, quantities, package size, and/or origin and destination. Consult your company's Hazardous Materials/Dangerous Goods expert information specific to your situation.

Section 8-Stavbility and Reactivity

STABILITY: stable

Avoid excessive heat or open flames. This material should not be mixed with phosphorous containing material or oxidizers.

INCOMPATIBILITY (MATERIAL TO AVOID):

Can react Vigorously with strong oxidizing agents and phosphorous containing materials.

HAZARDOUS DECOMPOSITION OR BY-PRODUCTS:

carbon monoxide and carbon dioxide.

HAZARDOUS POLYMERIZATION:

Section 9-Transportation

Will not occur.

DOT (DEPARTMENT OF TRANSPORTATION)

TECHNICAL NAME : Paint

UN NUMBER : UN1263

UN PROPER SHIPPING NAME : Paint

TRANSPORT HAZARD CLASS : 3

PACKING GROUP : PGIII

MARINE POLLUTANT : Not considered a marine pollutant.

SPECIAL PRECAUTIONS : Shipping instructions and descriptions may vary based on mode of transport, quantities, package size, and/or origin and destination. Consult your company's Hazardous Materials/Dangerous Goods expert information specific to your situation.

Section 10-Regulatory Information

UNITED STATES

SARA TITLE III (SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT)

311/312 HAZARD CATEGORIES:

See Section 2

H320 Causes Eye Irritation

H315 Causes skin irritation

H371 May cause damage to organs

H304 May be fatal if swallowed and enters airways.

H310 Fatal in contact with skin

HAZARDS IDENTIFICATION

CLASSIFICATION : Category 2 Skin Irritant

Category 2 Eye irritant

Category 3 Specific Target Organ Toxicity

Category 3 Flammable Liquids

Category 3 Acute Toxin-Dermal

SIGNAL WORD : Danger

HAZARD STATEMENTS :

Flammable liquid and vapor.

Toxic in contact with skin.

Causes skin irritation.

May cause drowsiness or dizziness.

Causes eye irritation

PICTOGRAMS

PRECAUTIONARY STATEMENTS :

Do not handle until all safety precautions have been read and understood.

Avoid breathing dust/fume/mist/vapors/spray

Wash thoroughly after handling

Wear protective gloves/protective clothing/eye protection/face protection.

Do not handle until all safety precautions have been read and understood.

Avoid breathing dust/fume/gas/mist/vapors/spray.

Wash thoroughly after handling.

Wear protective gloves/protective clothing/eye protection/face protection.

Keep away from heat/sparks/open flames/hot surfaces. - NO SMOKING

FIRE : This product is considered a flammable liquid.

PRESSURE GENERATING: No applicable information was found.

REACTIVITY : Slight reactivity. Avoid strong oxidizing agents.

ACUTE : No applicable information found.

CHRONIC : No applicable information found.

302/304 EMERGENCY PLANNING

EMERGENCY PLAN: This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations and the MSDS contains all the information required by the Controlled Products Regulations.

OTHER REGULATION : Not applicable

Section 11-Toxicology Information

| BUTYL ACETATE(000123-86-4) |  |
| --- | --- |
| Acute Oral Toxicity | LD50: 10768 mg/kg Rat |
| Acute Dermal Toxicity | LD50:17601 mg/kg Rabbit |
| Acute Inhalaltion Toxicity | LC50: 2000 4h Rat |
| Acute Oral Toxicity | LD50: 3500mg/kg Species Rat |
| Acute Dermal Toxicity | LC50: 17800 mg/kg Species: Rabbit |
| Acute Oral Toxicity | LD-50 Rat 1300 mg/kg |
| Acute Oral Toxicity | LD-50 Guinea Pig 1400mg/kg |
| Acute Dermal Toxicity | LD-50 Rat >2000 mg/kg |
| Acute Dermal Toxicity | LD-50 Guinea Pig >2000mg/kg |
| Acute Inhalation Toxicity | LC-50 Rat 3h >4.9mg/l |
| Acute Inhalation Toxicity | LC-50 Guinea Pig 1h >3.4mg/l |
| METHYL ETHYL KETONE (VOC)(78-93-3) |  |
| Acute Oral Toxicity | LD50 2737mg/kg Rat |
| Acute Dermal Toxicity | LD50 6480 mg/kg Rabbit |
| Acute Inhalation Toxicity | LC50: 32000mg/m3 4h Mouse |

Section 12-Special Precautions

Avoid contact; wear protective clothing and wash thoroughly after using.  Avoid inhalation of vapors; use only with adequate ventilation.  Do not take internally.  Avoid storage areas of excessive heat or near sources of ignition.

Description of first aid measures · General information: Symptoms of poisoning may even occur after several hours; therefore medical observation for at least 48

hours after the accident. · After inhalation:

Supply fresh air. If required, provide artificial respiration. Keep patient warm. Consult doctor if symptoms persist.
In case of unconsciousness place patient stably inside position for transportation.

After skin contact:
Immediately wash with water and soap and rinse thoroughly

Immediately remove any clothing soiled by the product.

If skin irritation continues, consult a doctor. · After eye contact:

Remove contact lenses if worn.

Rinse opened eye for several minutes under running water. Then consult a doctor. · After swallowing:

Rinse out mouth and then drink plenty of water.

Do not induce vomiting; call for medical help immediately.

EYES: Flush eyes with water for at least fifteen minutes and consult a physician.

SKIN: Wash affected area with soap and water and remove contaminated clothing promptly.

INGESTION: Do not induce vomiting. Never give anything by mouth to an unconscious person. Consult a physician.

INHALATION: Remove victim to fresh air area and administer oxygen if necessary. Consult a physician if necessary.

Other Precautions:

Keep out of reach of children.

NFPA CODES

3

0

2

B

| HMIS RATING |  |
| --- | --- |
| Health : | 2 |
| Flammability : | 3 |
| Reactivity : | 0 |
| Personal Protection : | B |

REVISION INDICATOR : Not Relevant

MANUFACTURER DISCLAIMER :  The information in this MSDS was obtained from sources which we believe are reliable.  However, the information is provided without any representation of warranty, expressed or implied, regarding its accuracy or correctness.

Section 13 Other Information

DISCLAIMER: The information Contained herein is based on the data available and is believed to be accurate, However, the manufacturer makes no warranty expressed or implied regarding the accuracy of this data or the results obtained from the use thereof. Accordingly, we assume no responsibility for injury from the use of this product.

Name:  Craig Larson

Title:  Chemist

Preparation Date:  11/01/2017